# Worklog

The impala data was collected by Chris Clemente, Jasmin Annett, Robin Maag, Chris Bird, and Taylor Dick across 3 field deployments in South Africa: 2024, April 2025, and July 2025.  The idea behind the project was to put our custom-designed collars (including an Artemis board which logged GPS and accelerometer data, as well as a stand-alone Axivity) on impalas, record their behaviours with a drone, and then reconstruct their exact behaviours and paths from the acceleration data. Luke Jessup and Jojo Schultz begun the process of understanding the dead reckoning workflow. The following document now outlines the application of the dead reckoning analysis as well as behavioural model by me (Oakleigh Wilson). Annotations done by Amelia Nelson and data crunching assistance by Ethan Norris.

I like to take notes as I work. These are things I observed and decisions I made. It is for my own sake of thinking-out-loud more than anything.

Aligning Accel and GPS	1

Dead Reckoning	5

Checking Data Scale and Orientation	29

Training the model	33

Integrating with Dead Reckoning	36

What I learnt and Workflow for next time	37

## Aligning Accel and GPS

### 18/03

- Some history to this project is that I started it when guiding an undergraduate student through preliminary energy landscape analysis. During this time, we realized that the “perfectly time synchronised” boards were, in fact, not time synchronised. ChrisC and I spent several weeks trying to align the board with the associated behavioural videos. That work is detailed in this googledoc: here. The long-story short of it was essentially that:

  - Extracting the data from the boards wasn’t straightforward. Once it’s done, it’s easy, but figuring it out for the first time took a lot of trial and error.

  - The time synchronisation video had been filmed with a camera never used for data collection (thus rendering it useless for time synchronisation)

  - The cameras that were used for data collection all had different ways of encoding the times  3 cameras filming the same event reported different times. Which time zones were these in (UTC, Australia/Brisbane, Africa/Johannesburg, daylight savings or no)? Do the time stamps refer to the start or the end of the video?

  - At the time, I was only able to achieve alignment for the drone videos as their timestamp was recorded in the filename.

  - Amelia Nelson (research assistant) painstakingly annotated every possible second of recorded drone footage. However, even after all her work, due to multiple device failures, this nevertheless resulted in only 2 individuals worth of data… which is insufficient for behavioural analysis.

  - Therefore, many months later, I’m back at it.

- First step is for me to get my head around the data that we have.  Have filled in the metadata sheet in the excel sheet in the Notes folder.

- What this tells me is that my next most valuable endeavour is to figure out the alignment from Robin’s camera. Going to have a crack at that this arvo.

  - Note that a lot of the code I wrote for the kangaroo data was for accounting the errors in the boards. As far as I can see, these same errors don’t occur in the impala data. However, I left the debug in for the impalas just in case.

### 19/03

- I hate myself. Why did I change so many things all at once???

  - Below is collar 8 video 2 as annotated by Amelia with the old system (looked correct to the video). Now I have to be able to replicate that with my new system.

  - Which I was unable to do… Which means that, somewhere my new system is breaking. Which is actually a good sign… because the error is in my new code, not in the data (flow on implications for why the kangaroo stuff isn’t working).

- I need to debug this systematically, working backwards.

  - I reverted all my changes to the plotting method and got the same result. Therefore, either the video times are incorrect or the gps times are incorrect.

  - I rechecked the video times and no joy. Therefore, I think the gps times are wrong. Going to revert all the work I did yesterday back to the original versions.

    - Note that the kangaroo data had a lot of error not encountered in the impala data (e.g., multiple board resets, skipped reads, incorrect GPS writes, etc). The fact that using the “cleaning code” I wrote for the kangies on the impala caused the impala to break indicates that there is something wrong with the cleaning code…

- Okay, I’m actually going to revert everything. Back to the very beginning and check differences along every step of the way. I created a branch from 5 months ago and will check every step along the way:

  - GPS stitch:  The old version misses the first GPS point for some reason? Moved to the new version.

  - Accel stitch: Exactly the same

  - Old method doesn’t account for the internal clock restarting on the accelerometer but when I more closely investigated, it does restart. There are duplicated values in the timestamps.  there is also a restart in the GPS

  - Found a section in the interpolated times where the clock started going backwards presumably because the GPS glitched and went backwards.

    - Reordered the dataset by the reset events and then removed the misreads where there was an uneven increment between the internal and external times on the gps_board

### 30/03

- Have been very busy teaching and submitting other papers. Am now back to working on this and feel like I’m going to suffocate from the stress of it.

  - Something I’m being wary of is the conversion between timestamps.

  - Below confirmed that the gps timestamps stay the same after writing and reading but also converted everything to numeric to be safe.

- First thing is to prevent the GPS from going backwards… Began by accounting for the reset events so the times aren’t matched across resets.

- This immediately showed where the interpolation error came from because all the error arose before the sampling even began (began on 28th, errors were back-interpolations to the 25th)

- But somehow they are then getting back mixed together after I combine the accel and gps?

  - I didn’t want to have to do this, but I might have to delete any data from the short reset events because they got blended… Left is before deletion, right is after.

- This should now mean the data is accurately aligned for interpolation… As far as I can tell, it appears to be…

- LADS we did it!!! I got it aligned!

- Now going to trial it on the kangaroo data and see whether this new code works on that dataset too.  it didn’t

### 31/03

- Today I need to align Robin’s cameras before Amelia gets in. Rush!

  - Wait. Wait. It seems like it might already be aligned? If you watch a video a sufficient number of times, you can gaslight yourself into literally anything. (Robin Vid 8 Collar 8 seems like it might be -10, Vid 9 is unusable…)

  - Noticed that the drone is aligning when I think both the drone and the board are in UTC time. Does this mean that the drone is in UTC (not the assumption I made for the kangaroo data, or that the board is in local time??) yes, they turned out to be different

- Amazing! So it was aligned and Amelia did a tonne of work on it.

### 01/04

- I quickly wrote up an instructions doc for how to do the alignment (VideoAlignment_Instructions.docx). Now doing one for sanity checking (SanityChecking_Instructions.docx) while I sanity check Amelia’s annotations so far.

  - As I was doing this, I found the ones that we did yesterday are scaled incorrectly and have been massively reduced… I vaguely remember that in the code. Okay multiplied them by 2048 which brought It back into the correct scale.

## Dead Reckoning

### 05/04

- I have decided I want to lock in and finish this paper before I leave for Germany.

- I feel really unwell but trying to do a little of this before I go to sleep. Trying to get the dead reckoning script set up for Ethan to work on in a fortnight.

- Quite quickly hit a problem where the calibration event occurred before the first reset, so wasn’t saved Unreal. How am I going to extract that if I don’t know the right time??

- Went and got the chunk from last time I did this (before proper alignment) so I can keep working for now.

- This is a segment from day 3 (collar 8). This is even the smoothed value… Clearly not cleaned enough. Need to clean all the data before I calculate any computations on it otherwise it will be totally out…

- In the meantime, I just did a Z score normalisation so I can plot them all on the same axes. The below image is what I have to use to figure out the orientation of the collar.

| series | mean |
| --- | --- |
| RawAX.sm | -0.83235 |
| RawAY.sm | 1.88471 |
| RawAZ.sm | 2.85395 |
| RawMX.sm | -272.433 |
| RawMY.sm | 255.0555 |
| RawMZ.sm | -176.346 |
| pitch | 13.67805 |

  - So, from my understanding… (Refer to the next section below  notes I wrote previously and copied over from the Mobile Biomechanics Lab GitHub)

    - Using these variables, I can see that… Not much, tbh.

    - In the NED (North-East-Down) frame, static accelerometer readings reflect gravity (≈1 G downward = +Z in NED). So this might be NED with a lot of roll… could also be NWD?

### Notes on Dead Reckoning variables

This function takes as input:

- Raw tri-axial magnetometer data (mag.x,y,z)

- Tri-axial static acceleration data (acc.x,y,z) (the smoothed values)

- Marked events data (ME) specifying the period of the magnetic calibration period (as denoted by ‘M’ - any other input signifies data acquisition not part of calibration procedure)

- Method of correction: default 3. * For rotated ellipsoids use method = 2 (more influenced by noise or more spherical) or method = 3 (default). For simple orthogonal re-scaling use method = 1. For non-rotated ellipsoids use method = 4, 5 (x & y axis equal), 6 (x & z axis equal), 7 (y & z axis are equal) or 8 (spherical data).

- Use 3 if a good calibration period is present. If the calibration period is not great (only ‘some’ of a sphere is covered when calibrating), then use method = 1.

- Estimated pitch offset: Approximately how much was it rotated by? * algorithm = “standard” (standard euler angle approach) or “SAAM” (using quaternions)

We have most of these variables available in the data already, but there are some that we have to figure out. To understand the reference coordinate system for this analysis, firstly we have to understand what the axes represent. This analysis system assumes the North-East-Down (NED) system where the X axis will be the surge (forward and back, North and South), Y will be the sway (side to side, East and West), and Z will be heave (up and down).

We then need to orient this accelerometer inside of the broader world. We do this by describing the compass direction of each axis. To conceptualise this, think of your hand as a practice accelerometer. Make a gum pointer finger with the middle finger extended as well such that the 3 digits will be at right angles from each other. The index finger represents the X axis, the middle finger is the Y axis, and the thumb is the X axis.

In the above image we have imagined a compass. We then have to describe the direction of the 3 axes (fingers) according to these compass directions in the order of X, Y, Z. In this case it will therefore be North for X, East for Y, and Up for Z. The coordinate system would therefore be North-East-Up (NEU). In total, there 48 possible combinations of coordinate frame configurations.

The default orientation of the Gundog system is North-East-Down (NED) where the axes readings would be X = 0, Y = 0, and Z = -1. Before we could use data from our hand accelerometer we would have to flip it upside down. Some devices may have a different local coordinate frame, e.g., the END (East-North-Down) coordinate system assume the surge dimension of movement is represented by the z-channel, and the sway and heave dimension, by the x- and y-channels, respectively. Providing the original coordinate system allows the Gundogs system to convert your local system to the global system. The Gundog.Compass function comes with three inputs to allow the user to transform their device’s coordinate frame into the NED that the specific Direction Cosine Matrix (DCM) and quaternion augmentation uses to compute angles:‘acc.ref.frame’, ‘positive.g’, and ‘mag.ref.frame’

### 14/04

- Clearly something is wrong with my dead reckoning, it’s not even close to the GPS track. My assumption at this stage is that I have the local orientations incorrect.

- I am now working in the https://github.com/OakAlice/IntegratedCollarAnalysis code, working through this code  https://github.com/OakAlice/IntegratedCollarAnalysis/blob/main/Scripts/DeadReckoning/TestTrial_Report.Rmd which Jojo and Luke originally developed. I know that the test trial should result in multiple rectangles but my current trials are getting waves… I am now playing around with the different orientations until I get something good.

- This approach ended up being futile so my next step is to go back to Jojo’s original code and look for differences line by line.

### 20/04

- Last week ended up being completely devoted to figuring out what the problem with the kangaroo collars was. After many – many – hours of effort by Chris Bird and I, he determined that the devices were repeatedly pausing (so the internal clock not incrementing and no samples being taken) which is why the GPS and accel were out of alignment. Thank goodness this didn’t happen with the impala data (we checked). Therefore, this week, I can return to this analysis.

- I went back to the flat data collected by Luke & Jojo, and, using my existing code, tried to replicate walking around the football field. (following image) and it was way off. At least this is a good sign. I have done something completely wrong.

- Working through this is very frustrating and making me depressed again… I don’t have their original data and it is very hard to make sense of their original mess of code but here we go.

- Have gone right back to basics and am working through Jojo/Luke code.

  - I found their original data and it works perfectly. So now I need to remove all variation between theirs and mine until I can reliably replicate it.

  - Found the error – or at least one of them. My magnetometer has not been scaled in the same way as theirs has.

Their data:

My data:

  - I went through in detail and found that my conversion for my magnetometer are off… After aligning them exactly between the GPS points, I figured out that I needed to multiply the magnetometer by 0.15 to give me what jojo had. So…

mine.flat[, c("RawAX", "RawAY", "RawAZ")] <- mine.flat[, c("RawAX", "RawAY", "RawAZ")] / 2048

mine.flat[, c("RawMX", "RawMY", "RawMZ")] <- mine.flat[, c("RawMX", "RawMY", "RawMZ")] * 0.15

jojo_df <- df.flat[1:77009, ]

oak_df <- mine.flat[3064:80072, ]

  - OMG! That was the only correction I needed and now it aligns beautifully and looks exactly the same! Now to figure out when it isn’t flat.

- The flat results on the left and the 45 degrees on the right…

- Excellent. I am now confident that I have an understanding of how this works and how to use it. I now want to expand the workflow to be used on the impala data. The hardest part of this is probably:

- Figuring out the correct orientation of the devices and whether the accel and mag are in the same orientation…

  - I think I should presume that they’re not. That is, jojo figured out that the accel was NED and the mag was NWU. Whatever my accel orientation is determined to be, the mag will probably be an equivalent rotation.

- Extracting the calibration periods

  - I previously altered my workflow such that it discarded all data before the first sat hit. However, I subsequently realised that this discarded the calibration event. Therefore, I need to rework the impala alignment flow.

- By a stroke of good thinking, Jaz realised the orientation of the accels are written on the devices. Therefore, we were able to determine the orientations directly. Photo below for reference.

- I now need to apply the DR code with predicted orientation to a sample of the impala data and see how we go. Pray for me.

☒Ensure the calibration event is saved separately during the data reading process

  - Changing workflow to retain all reset events instead of only the longest ones.

  - Was having an issue where there were a lot of unmatched gps pings. Fixed this by switching the order of the kets for the rolling join. But I nearly kms before figuring that out.

  - We cooking with gas again.

- Okay now I’m finally up to the part where I have to decide the orientation of the device. Oh no.

  - This is the GPS I’m aiming for…

### 22/04

- Yesterday I walked the collar around in the approximate correct orientation. Unfortunately, I didn’t wait long enough to get a proper gps hit but otherwise collected data from 3 conditions (flat collar, 45-degree pitched collar, and messed up variable collar as an impala simulation). Today I am trying to reconstruct the paths.

  - Based on my best understanding of the collar orientation, I believe the collar should be: X heave, Y sway, Z surge. Therefore. That would translate to an orientation of… Down, West, North (DWN). According to the notes it should be END though.

  - If the original collar trials had an acc of NED and a mag of NWU then… I need to reverse the Y and Z directions for the mag of the true collar orientation. So would be… DES.

    - Okay that was appalling and not close at all. NED, NWU was much closer.

    - Need to use the method = 1 because we didn’t get proper calibration data.

Closest 45 degrees and then the same settings on the variable… (even holding the pitch and roll constant from the stable trial). HUH. This is not good. What am I doing wrong? Going to repeat trials tomorrow but over a longer distance so that we guarantee a GPS hit and I can scale these in more appropriately.

  - For now I am going to go back to debugging whatever was wrong with the files that Ethan identified yesterday.  Issue was that calibration event could not be found.

    - Basically, the calibration event occurs at 28.06.24 18:16:00 local time but the first recorded gps time is "2024-06-28 16:28:48 UTC" (about 10 minutes after). Jaz says its not possible for the calibration event to have occurred before the first ping… but stuff happens. That would certainly be the most parsimonious solution. But I will manually search for it in the acceleration data anyway in case something has gone wrong.

      - Turns out this is needle in a haystack behaviour.

  - The first data recorded by the accelerometer apparently starts just after midnight UTC. Which is only reasonable if it was in Australia, which it wasn’t.

    - Found that the accel was getting inappropriately cropped to start on the 30th not the 28th (first gps hit)

    - Yep. Even when I don’t crop the accel, the interpolation sends it forward in time from the first gps hit. Interesting. Where the time jumps backwards in the first gps hit. So what the heck went wrong with the reverse interpolation?

    - RTC accel (left) and the gps times look correct, however the conversion to the gps time using interpolation is way out (right) where it jumps several days.

  - Changed the way backwards times were calculated and it got back into alignment.

- Once I confirmed that the calibration was extractable, I automated this stage for every collar.

### 23/04

- I was meant to run the loop overnight but the code failed on the second line… killing myself.

- Ah. So. The GPS only had 1 reset but the accel had 2 and it cropped the accel too small… So. That is an issue. I have to change my joining logic.

  - Changed the gps reset_events manually over to the longest accel sampling period number.

  - Getting rid of the files with bad headers seemed to deal with that issue anyway.

- Now I am automating it through the remainder of the collars. Will talk to CB about DR.

  - Jk. The next collar ran into a new problem. It never ends with this BS.

    - While I can’t (so far) find any errors in Collar_12, the calibration period it extracted does not look like a calibration event… However the second calibration event (from after deployment --- does turn out perfectly… And the interpolation seems like it checks out. So idk what to think. I might just use the latter calibration?

  - Collar 13 had issues because it turned off after 44 hours. When I removed the 2022 files it worked fine otherwise…

    - So am I meant to remove these files or not? When I leave them in, there’s issues. When I take them out, it works… But they seeeeeem like they’re correct? Until we understand how they got there, I’m reluctant to do anything

### 27/04

- Ran another field trial this morning, walking around campus. This time I walked for 30 minutes to secure lots of GPS hits. Trial 1 (15 min) was flat collar. Trial 2 (15 min) was moving the collar all over the place but walking the exact same route (i.e., no doubling back or actual weaving)

- Interestingly, when the device was first turned on, it was trash. Lost most of the first trial because it was misreading.

- Compass visualization randomly worked for the first time ever??? And it looked trash. I don’t think I’m calibrating correctly…

- This was the route I actually walked:

- The reproduction from the dead reckoning is… not even close. Is it because my calibration was shit, or what?

- I am going to give up for now. I need to come back to this with a clear mind later.

### 30/04

- Today I am beginning locking in on the DR as I have decided I need this for my conferences coming up. I need to:

#### Read the email thread with Luke/Jojo and Richard

- “good mag cal (ensure not wearing watches /have phones/any magnetic interfering stuff on person etc. nearby when doing this)”

- “In terms of movement bouts, the simple approach would be to use some sort of acceleration magnitude threshold (e.g. VeDBA)... but for example, something like a kangaroo should be very obvious with jumping behaviour that should come out in acceleration traces as clear stylised patterns which could be manually discerned or via machine learning if you were able to do some groundtruthing of behaviours during the data acquisition period.”  by the sounds of this, only meant to do when moving… lol.

- “If your tag has a consistent minor offset (e.g., due to collar placement with a slight tilt), and you’re unsure about the exact amount, it’s usually not necessary to worry about correcting for this. The pitch and roll values from the device will reflect its orientation relative to the ground (gravity vector) and will automatically correct the calibrated magnetometer vectors before calculating the heading. If the collar rolls smoothly and independently of the animal, the same principle applies.”

- “When the penguin is walking, the tag will be nearly vertical, while during swimming, it will be more horizontal. In both cases, the tag’s coordinate frame relative to the penguin’s natural movement (and the ground or water surface) will be different. To ensure accurate heading calculations, I would need to define different tag coordinate systems for walking and swimming and compute the heading separately for each period—on land and at sea—otherwise, my dead-reckoned (DR) tracks could be very innacurate.”  no WONDER my flippy flippy stuff doesn’t work.

- “If you know there’s a specific yaw offset in the tag placement, you can specify it in the compass function. If you’re unsure, you could compare the heading correction factors from Gundog.Tracks. If the headings are consistently biased in one direction, it could indicate a tag yaw offset or the need for magnetic declination adjustment. But again, if the tag’s yaw offset is minor and consistent, it’s not essential to correct this, as the dead-reckoned track will be consistently biased (in theory, the patterns and scales of tortuosity between GPS points should remain the same, just directionally offset somewhat), and the track segments get rotated when corrected with their endpoint GPS fixes during the correction process anyway.”

- “What I suggest is to first run the compass function without any offsets applied to derive the pitch and roll angles. Then, determine the mean values of both during periods where you believe the animal and collar are in a normal position, such as during low-activity, steady-state behavior. Once you have these values, re-run the function and input them as the 'pitch.offset' and 'roll.offset' parameters.”

- “if you supply a positive pitch offset value of 45°, this reflects that the device is pitched up by 45° from the level, and the function will de-rotate the supplied (smoothed) acc and mag (normalised) values by this amount. Similarly, a positive roll value indicates a rightward tilt (bank angle), and a positive yaw value represents a clockwise rotational offset”

- Luke wrote this: “for the raw files, ~2048 LSB = 1 G, and the magnetometer data has a sensitivity scale factor of 0.15 (https://invensense.tdk.com/wp-content/uploads/2024/03/DS-000189-ICM-20948-v1.6.pdf)”  no way… lmao.

- “The short answer is, yes - if a wrong ref frame was chosen then DR path wil be innacurate. But comparing subsets with GPS pre-correction will help debug if chosen orientation was correct. The patterns and directions of travel should be similar even if scale is not when compared to GPS. I had some lions where I had to 'figure' out the orientation post release. But visually it should be easy to decipher the heave axis of movmement in acc and mag (acc will be the one closest to +/-1 g most of the time, and mag will the one changing the least)... In terms of deciphering the acc/mag surge and sway dimensions, this may be more challenging. But for the acc, you would expect greater variation in surge dimension because they stick their head to ground to forage and then stick their head back up when travelling and vigalence, so that may help. If mag is different orientation set up to acc, then its worth just checking if you switch the surge/sway orientations then which one produces most realsitic DR track relative to GPS... by deciphering the heave axis and dimension of measured g, you already have homed in on just a few possible orientations it could be...”

- Holy flip. These emails explain everything. I should have read these a month ago instead of wasting all my time. It’s incredible what one can learn from the handover documents!

### 01/05

#### Read Dead-reckoning animal movements in R: a reappraisal using Gundog.Tracks

- On its own the dead-reckoning process is prone to cumulative errors, so that position estimates quickly become uncoupled from true location. Periodic ground-truthing with aligned location data (e.g., from global positioning technology) can correct for this drift between Verified Positions (VPs).

- Verified Position Correction (VPC) dead-reckoning in R using the tilt-compensated compass method

- Dead-reckoning is a method used to reconstruct animal movement paths, based on sequentially integrating the vector of travel from a predetermined position using estimates of heading (also termed ‘bearing’ or ‘yaw’) and speed (and displacement about the vertical axis for 3-D movements), over an elapsed time interval

- Of the above VP options, GPS-corrected dead-reckoning is the most widely used

Gundog.Tracks(TS, h, v, elv = 0, p = NULL, cs = NULL, ch = NULL, m = 1, c = 0, ME = 1, lo = 0, la = 0, VP.lon = NULL, VP.lat = NULL, VP.ME = FALSE, method = NULL, thresh = 1, dist.step = 1, bound = TRUE, Outgoing = TRUE, plot = FALSE)

  - See image below for all the possible variables.

    - Note that Marked Events (ME) is meant to denote when the animal is and isn’t moving. 0 for stationary, and 1 for movement.

      - The user may also opt to supply a ‘marked events’ (ME) vector (a marked event is a term we use that refers to a number of sequential (in time) data points within a dataset coded by integer values) to ensure dead-reckoned tracks are not advanced with non-animal movement behaviours.

      - NOTE: If we have been doing this wrong in our trials, why did they work? I should switch that over and see what happens…

    - high-resolution VP data are available (e.g., 0.01–10 Hz GPS)  okay, nevermind, we aren’t high frequency enough for this to matter

    - VP.ME when TRUE removes the GPS points when the animal is not moving.

    - Method == “all” will retain all GPS points (rather than selecting only a subset)

  - Within Gundog.Tracks, the user can modulate m, c and ME values to switch between predetermined speed (m = 1, c = 0), DBA-derived speed (m > 0, c ≥ 0) and stationary behaviour (ME = 0).

  - Here an an example from his data that shows the expected error… if he’s using it in the paper then it must be considered good:

    - This represents 2 weeks of lion data where GPS was sampled 1/sec. That’s not an insignificant amount of required re-correction… Keep this in mind for how much error I’m encountering.

  - VP error becomes more relevant at smaller scales of assessed movement and this is the reason why VP distance-moved estimates can go from being typically underestimated at low frequencies (due to linear interpolation of tortuous movements) [26, 140, 141] to overestimated at high frequencies [97, 136] and result in highly variable correction factors within the VPC dead-reckoning process

  - Put simply, one should VP correct as little as possible, but as much as is necessary

  - For collared animals, heading measurements can become inaccurate at times of erratic collar roll

  - For example, the benefits of dead-reckoning a lion at 40 Hz rather than 1 Hz are questionable (how often does a lion turn substantially within a second?)

    - Should I be downsampling my accelerometer? Have I missed that?

  - Preparing the three axes of rotation for derivation of heading

    - Correct coordinate system axis alignment and suitable calibration of tri-axial magnetometry data are crucial pre-processors, without which, heading estimates would likely incorporate substantial error

    - The tilt-compensated compass method requires the aerospace (x-North, y-East, z-Down) (right-handed) coordinate system, or ‘NED’

    - Device orientation is expressed in terms of a sequence of Euler angle [roll (Φ), pitch (θ), yaw (Ψ)] rotations about the x-, y- and z-axes, respectively, relative to the (inertial) Earths fixed frame of reference

    - Being a vector field sensor with two degrees of rotational freedom, accelerometers are insensitive to rotations about the gravity vector and thus discerning heading requires the arctangent of the ratio between the x- and y-orthogonal magnetometer measurements [156]. For the correct computation of heading, these two channels need to be aligned parallel to the earth’s surface. This is achieved by correcting any orientation (de-rotation) according to pitch and roll angles (postural offsets) which can be deduced from acceleration.

    - Importantly, for correct trigonometric formulae output within the tilt-compensated compass method, the vectorial sum of static acceleration and calibrated magnetometry measurements across all three spatial-dimensions must be normalised (to a unit vector) with a scaled magnitude (radius) of one

      - This occurs within the function

    - Also need to smooth the mag data

  - We recommend implementing a running mean (cf. Eq. 1, R1:4) to raw VeDBA values to ensure that both acceleration and deceleration components of a stride cycle are incorporated together per unit time and to reduce the magnitude of small temporal spikes

  - For similar reasons, it is also worth post-smoothing raw pitch, roll and heading outputs, although heading requires a circular mean

  - You can create different speed algorithms once you’ve separated walking and running from the other kinds of movement / non-locomotion

#### Read the Supplementary information he provided

- It is essential that tri-axial accelerometer and magnetometer readings are aligned to a designated tag coordinate system for accurate delineation of 3-D rotation with respect to the fixed Earth’s surface.

- For NED:

  - When lying flat and upright, the z-axis reads approximately 1 g (1 g = 9.81 m/s) and the x- and y-axes are near 0 g

  - Assuming the magnetometer and accelerometer channels are in alignment, the x-channel magnetometer reading is at a maximum and minimum when the (whole) device (positioned flat) is pointed North and South, respectively. The y-channel is at a maximum when the device is rotated to point West and reads a minimum when the device is pointed East, with the z-channel reading remaining relatively constant. Conversely, the y-axis will read a maximum when the device is pointed East following a 180o inversion, with the z-axis reading (though still constant) being lower than the upright condition (in the Northern hemisphere), because the channel is aligned against the downward geomagnetic field vector.

  - Using this local frame, we assume that the x, y and z axes of both the accelerometer and magnetometer sensors are aligned to the anterior-posterior (surge), medio-lateral (sway) and dorsal-ventral (heave) axes of a given animal, respectively.

#### Do the Penguin worked example provided in the paper

  - Downloaded the txt file and code from the supplementary materials.

  - In his worked example, the points turned up on the compass again, which would indicate that I’m doing something wrong with mine to make them not appear…

  - Was also able to recreate his results no issues…

#### Back to my data

- For example, day 3 of the impala data GPS was below… surely this is starring at least a bit? I have no reason to think these are super high precision actually. I should smooth these as per usual.

- More importantly, I need to figure out the orientation. So from Gunner’s paper, the z axis should be +1 when the device is completely flat. I need to figure out what it is for us…

  - Based on inspection of the actual collar, looks like the Y axis is completely up, X is side to size, and Z is back and forth. And then the Magnetometer is different in that Y is straight down when flat.

- JUST REALISED I can use the data that Amelia already annotated rather than searching through! So in the walking data (left) and vigilance data (right) from Collar 8!

  - The orientations of the axes as written on our board are as follows.

| X acc | Side to side | Sway | East |
| --- | --- | --- | --- |
| Y acc | Up and down | Heave | Up |
| Z acc | Forward and back | Surge | South |
| X mag | Side to side |  | East |
| Y mag | Up and down |  | Down |
| Z mag | Forward and back |  | North |

  - I then tried to verify this by looking at the accelerometer data when the animal was standing up (ground-truthed with video). Red is X, Blue is Y, Green is Z. When the animal is standing up, the X axis has close to 0 gravity. The gravity is shared over the Y and Z, with the most on the Z. This indicates that the collar is tipped forwards.

| X | -0.160 |
| --- | --- |
| Y | 0.633 |
| Z | 0.729 |

  - Therefore, I propose the orientation is: acc EUS, mag EDN, gravity down, and a pitch forward of about 48 degrees…

- After all of that, I still got a nonsense result:

- Shooting myself?

- No. I need to lock in and do what needs to be done which is actually clean the data and get rid of the massive misreads. Because errors are cumulative, my assumption that these errors wouldn’t affect it are probably wrong.

- From my understanding of the paper, some things I need to do are:

  - Remove outliers in the GPS

  - Remove outliers in the accel data so we don’t get VDBA peaks

  - Remove the magnetometer misreads… it’s going crazy for some reason. Need to pull it back in.

### 05/05

- Happy birthday to me. Also, today is the day that I try removing outliers from the impala data – something I should have done over a month ago. Of course it could never work when there are such large errors throughout

  - Often I see people using Butterworth in literature but searching more specifically I’m tuning up lots about the Kalman filter

  - The noise I want to remove is specifically impulse noise with the huge random spikes.

    - I have seen information about the Median Filter come up… trialing that now.

    - Here I have plotted the original axes and (a different section of) VDBA and then the median filtered one.

  - Similarly it occurred to me that I should be cleaning my GPS of noise after all:

- Now when I run my DR code… for the first time, I’m actually within spitting distance. Though I do note something funny in my IMU data (circled in yellow).

### 06/05

  - What am I going to do about that funny section? I’m going to find it in the data and plot that time period to see whether it looks obviously weird.

- I zoomed in on 2 weird parts, 1:500000 and 2850000:3500000. Below image outlines what it looked like.

- I have put message in chat to get ChrisB thoughts but in the meantime I will work towards what I think the next step will be: finding all the misread sections and removing them. Similar to the kangaroo data (though that’s worse), we have to go through and chop up the data into good sections with NA’s in between?

  - So, some things to keep in mind:

    - I don’t want to remove sleep

    - I want to tag when the animal is moving or not (based on ML, to be done later)

    - So maybe before I go deleting everything, I can just tag these sections as no movement?

      - I can go through and assign the Marked Events (ME) 0 for stationary, and 1 for movement. And put these flat bits into 0.

- Plotting all 3 axes sheds a bit more light on it as I can now see what when the device is misreading, the Y and Z axes are inverted. Head up, head down?

- I pulled out the annotated data that Amelia made and had a look at various behaviours. The collar angles are a guess, but the lines show the general shape of the data in the behaviours she observed.

- Then ChrisB pointed out that this was happening over hours not minutes, and is therefore more likely to be sleeping. He said “In that static accelerometer region it's likely just measuring gravity, but if the rotation of the collar isn't pointing the right way it will see that as an acceleration in the wrong direction and pick that up as movement”. Ahhhh brilliant man has saved me once again. So if I can detect that as sleeping then I can set to no movement and no DR during that period.

  - Back to what I was doing detecting the flat bits… The way I did that in my VDBA paper was that I found a place where the standard deviation wasn’t changing. So I will try that again…

  - Note that this will be easier once I have built the behavioural detection model. However, for now I just picked a threshold that looked okay. (above  ifelse(VDBA.sm > 0.015, 0, 1) & most common level in 60 seconds (3000 samples))

  - Testing that however showed no difference and the gps is moving consistently through the period???

    - I then isolated just the part where my accel says the GPS isn’t moving… and still got a displacement of 30m with the smoothed values for the black part of the line. The total “distance” looks like maybe 50 – and that’s after smoothing. So I think I’m going to have to add a step where if the animal hasn’t moved, take the average value. (Like I did for the koalas).

- Epic I worked on that for ages just for it to be worse… Lmao. I don’t know why it’s so much worse. Calling it for the day unfortunately but better luck tomorrow :/

### 07/05

- I’m hoping the bad tracks from yesterday are just some kind of code error and that I’ll be able to clean it up enough today to send through to Gunner asking for help.

- Firstly I continued with the cleaning by adding a lowpass butterworth filter with a cutoff of 10Hz. The results of the 3 steps of smoothing shown in right plot below.

- Current GPS smoothing not aggressive enough. I need to make sure the periods of stationary have all their GPS grouped together…

  - For example, in this case, there are 2 stationary periods where there should be no GPS changes. The dark blue dots in the gps graph right show the stationary periods. The green dots are what got fed to the spline and the red is the spline path. Note there are only 2 black dots because all the stationary got grouped.

  - But then on the next example it broke and make a silly line…

  - Found the error in how I was calculating the time. Now that has been corrected and results are more like:

- I figured out that we had not included the ME line in our call. When I added that in, the flat straight sections disappeared, yay. However, it is still clearly very wrong. Lmao. If you don’t laugh, you’ll cry.

- Then ChrisB helped me for a few hours, figuring out many things. For example, he found that everything was scaled by 4 because the settings of the accelerometer weren’t what I thought. (They were 16 for the kangaroos but 4 for the impala)

  - “Accelerometer scaling of 2048 LSB/g assumes the full-scale range is set to +/-16g. This is (should be) the case for the kangaroo data, but I don’t believe is the case for impala. Impala may have been at +/-4g.” ~ Chris Bird… and see fact sheet below.

- Note that this also gives us the noise bandwidth of 10Hz

  - Chris Bird also doesn’t believe in tilt-corrected accelerometer based DR and thinks we should have to use the gyroscope (pronounced guyroscope… as I learnt today)

- Okay. What to try next?

- I went and pulled the following quote from the emails between Gunner and Jojo. I should try doing this rather than seeing it from the collar.

  - The short answer is, yes - if a wrong ref frame was chosen then DR path wil be innacurate. But comparing subsets with GPS pre-correction will help debug if chosen orientation was correct. The patterns and directions of travel should be similar even if scale is not when compared to GPS. I had some lions where I had to 'figure' out the orientation post release. But visually it should be easy to decipher the heave axis of movmement in acc and mag (acc will be the one closest to +/-1 g most of the time, and mag will the one changing the least)... In terms of deciphering the acc/mag surge and sway dimensions, this may be more challenging. But for the acc, you would expect greater variation in surge dimension because they stick their head to ground to forage and then stick their head back up when travelling and vigalence, so that may help. If mag is different orientation set up to acc, then its worth just checking if you switch the surge/sway orientations then which one produces most realsitic DR track relative to GPS... by deciphering the heave axis and dimension of measured g, you already have homed in on just a few possible orientations it could be...

  - Below I have included some of the walking data Amelia annotated, and some grazing

  - From these graphs I can guess that the heave axis is the Z axis. The Surge is the Y. And therefore the sway is the X.

    - This is different from what I thought it was before. I previously thought it was EUS and now I think it is… ESU or END or WNU or WSD in which case the magnetometer would be… END, ESU, WSD, or WNU

    - Guessing and checking with Chis Bird eventually generated the below which he thought was the closest… I wasn’t really convinced.

      - acc_orientation <- "NWU",   mag_orientation <- "NED",   gravity_direction <- "down",   pitch <- 0#-48

  - I think it might be time to ask Gunner for help? I’m going to write an email tomorrow.

### 08/05

- I wrote up a massive email and document. In the process of which I experimented with more variables and got it closer by switching the method of compass from 3 to 2 (better for when there is a lot of noise) and better calculated the pitch from walking only events.

- See the “FiguringOutOrientation.docx” and email to him for more information.

## Checking Data Scale and Orientation

### 12/05

- Amelia has officially finished annotating all the labelable data but… unfortunately, it really just isn’t that much data. I’m going to need to generate more of it. Specifically, I want to know the difference between head up and head down behaviours to improve upon the DR. Below is what we have so far… Today I am going to try to get Ethan to increase it by finding new samples by blind searching.

  - Specifically I want specific behaviours. Will make a guide to them so he knows what to look for…. Those are on the next pages.

X = "salmon", Y = "turquoise", Z = "darkblue"

- Ethan and I disagreed for a while about which behaviour was what but ultimately, I realised he was completely right. The pattern as far as we can see is as below:

  - When the lines are flat, they’re just chilling, when the lines are wiggly it’s foraging, and when there’s a lot of mess and overlap it’s walking or a more complex behaviour.

- However… just a heads up, it seems that the devices were all scaled differently. Lmao. This will need to be controlled for in both behaviour searching and the dead reckoning.

  - I maybe need to reprocess all my impala so they’re all consistent. And then I can find time matches to the annotations to have the correctly scaled behaviours to make that easier.

  - Plotted it to see what units the devices are in and holy fuck, why are they all different??? What evil tomfoolery is this?????? Messaged ChrisB to ask what the heck is going on.

- And to make it even more epic, some of the orientations seem to be different. For example, Collar 6 has Y at the bottom when walking. Wow. Okay so fuck me. Back to the beginning. I need to standardise all of these before I can progress.

- Therefore, I am now working on converting all the scales to the same and then getting the orientations in alignment.

### 13/05

- Turns out the settings are in the OLA setting config file!  Except not all collars have it… and, on closer inspection, it’s clearly not true. These are the settings as reported by the config files but we can clearly see this is not the case in the graphs from yesterday.

- Therefore, go back to the code from yesterday and try to determine it manually again. UGH.

- Basically I found that there are 3 settings. Either the device is already in G (either 16 or 4) or its in raw units and has to be converted. The cropping at 4g means I am going to have to match and join it to the Axivity 😢ChrisC says this code exists and already works… but I’ve heard that often enough to know that means “this will be hard”.

| Collar | avg_hz | Acc_Conv | Mag_Conv |
| --- | --- | --- | --- |
| Collar_11 | 49.41162 | /2048 (then +/- 16g) | /2048 then *0.15 |
| Collar_12 | 49.48353 | /2048 (then +/- 16g) | /2048 then *0.15 |
| Collar_14 | 49.67822 | /2048 (then +/- 16g) | /2048 then *0.15 |
| Collar_15 | 49.9235 | +/- 4g | *0.15 |
| Collar_2 | 49.64591 | /2048 (then +/- 16g) | /2048 then *0.15 |
| Collar_3 | 49.62805 | +/- 4g | *0.15 |
| Collar_5 | 49.67374 | +/- 16g | /2048 then *0.15 |
| Collar_6 | 49.65929 | +/- 16g | /2048 then *0.15 |
| Collar_7 | 49.54214 | +/- 16g | /2048 then *0.15 |
| Collar_8 | 49.84833 | +/- 4g | *0.15 |

- Tried to figure out the orientation of each of the devices by plotting some random bits of it and seeing whether they all had the same basic patterns.

- Manually plotted and explored each collar. Kept notes here.

| Collar | Mean X | Mean Y | Mean Z | Screenshot |
| --- | --- | --- | --- | --- |
| 12 | -1.714332 | 0.691165 | 2.963385 | accel_data[1445000:1452000] |
| 2 | -0.0630 | 0.83789 | 2.9980 | accel_data[1660000:1670000] |
| 3 | -0.498123 | 0.1800641 | 0.7116897 | accel_data[90000:91000] |
| 5 | -1.2077 | 0.76058 | 3.1759 |  |
| 6 | -0.2416 | 1.2828 | 3.0252 |  |
| 7 | -1.8772 | 1.5167 | 2.5114 |  |
| 8 | -0.1593 | 0.2151 | 0.7584 |  |
| 11 | -0.5326 | 1.0954 | 3.10699 | There are also brief sections of this trace where the collar enters an unfamiliar orientation. (As per below where the Y axis has more acceleration than the Z axis) |
| 14 | -1.805006 | 1.146673 | 2.655918 |  |
| 15 | -0.458570 | 0.311943 | 0.62460 |  |

  - Okay cool, from that I conclude they are mostly in the same orientation for most of the same time, so I can skip the reorientation step.

  - Therefore, I went through and made sure they’re now on the same scale (other than the ones that capped at 4g and therefore have to be matched with the Axivity before behavioural and VDBA analysis). I then re-extracted the training data for those periods based on the times of the original unsmoothed data. That code is now running. I will check the output tomorrow.

### 14/05

- After all that conversion work, it wasn’t correct… They did need to be divided after all. Weird? Message CB and he said that things can change or if it reset/rebooted it might have gone back to default. Whatevvvvvvver man. I can’t keep up with all this. Converted everything to the right scale for real now and then proceeded.

## Training the model

### 15/05

- Yesterday I extracted a tonne of data and checked it all. Looks mostly good. Some errors I can see, but these will be cleaned out in the next step of the second matlab… Which, now I think about it, should be done before I stitch back to the original behaviours… Going to rearrange the code now.

- Well… I’m not sure what I did, but I completely messed it up and the data is unusable now. Fortunately, I still have all the original files, so nothing that can’t be undone, but I really don’t know how I cooked it so badly. Trying to find the bug in my workflow now…? But otherwise, other things I noticed:

  - Split out “Vibrating” as its own event because it looked weird.

  - Couldn’t find much head up foraging

  - Not a lot of sleeping, weirdly…

- Okay I fixed the error but there are still problems with the annotations… I think I should sort them into their final group before I clean them. Ugh. Back to the beginning! As long as this is complete by the end of the day and I’m generating features over night, I think I can afford to lose an entire day on the task… In the scheme of things.

- About to crash out and give up, this is so frustrating.

- For example, this behaviour has sometimes been labelled as foraging, sometimes as walking… It is really unclear what it should be. So it is definitely going to be mixed up in the analysis. My DR is going to need some hard bounds about head up/down, and then leave the VDBA to figure it out?

- The result I ended up with… is not perfect but I genuinely cannot go back through all of that again. Just going to call it for now and see how I go.

- Need to build in some cleaning logic for when the collars flip… ie. When the Y axis is on average higher than the Z axis, remove as a collar flip.

- And then logic the dead reckoning based on the behavioural analysis.

### 19/05

- I ran the feature generation last night but forgot to change one of the variables so it only generated 63 features instead of all of them. Ugh. And it took all night so I can’t afford to run it again right now. Have to remember to run it again tonight… In the meantime, will prepare the next phase of code.

- The models are doing reeaaaaallly badly with the limited features. Even the RF only got 26%.

  - From what I could see, the confusion between grazing_headlevel and walking was total. I collapsed those classes into one. There was also a lot of confusion between vigilance and vibrating. I also collapsed those categories together as well. I am rerunning the rest of the feature generation overnight and will check again tomorrow.

### 20/05

- Looking at the behavioural model results so far, there is such a massive confusion between some of the classes. Before I collapse the classes, I want to try training a more powerful model: 1) XGBoost, 2) a basic NN.

  - The XGBoost is doing terrible, with a below chance performance. I must be doing something wrong in the code…

- Currently fighting for my life to get keras working so I can use deep learning models. I want to try the NN at least once before I move on.

  - Gave up on keras and used neuralnet a simpler version instead.

- Head up

## Restarting the Dead Reckoning

### 21/05

- I have been working on the kangaroo data this morning and trying to get the dead reckoning working given the dual constrains of a) no calibration data and, b) broken/discontinuous data. Results so far are trash…

- With so many variables its very hard to determine why these results are so wrong. Going to now go back to the impala data and trial the DR without the calibration (just using the provided quaternions).

- If this doesn’t work/improve the method on the impala so far, then I am going to switch Dead Reckoning methods. Currently, due to the impala changing head orientation for the two main types of locomotion (walking and grazing/walking), the DR is massively overestimating the distance travelled during grazing (as the forward axis experiences a surge of gravity  without the gyroscope, this reads as a sprint forward…). While my initial intention was to do DR in iterative stages (head up… head down… head up again, etc…) but it is not at all evident how I would go about that. Instead, it will be easier to switch to a DR approach that natively includes the gyroscope.

- Oh wait hold up!

  - Chris Bird said that the quaternions in the impala data are yes there but are probably wrong because a library wasn’t working that he had to fix…

  - “In original IMU firmware library, setting the digital motion processing (DMP) reverts back to a lower accelerometer full scale range (FSR), despite this being set to +/-16g. This firmware library has since been updated in the kangaroo data, to allow DMP to run at the higher FSR” ~ Chris Bird, 2026

  - Never mind then.

- He then went through a few links and found a gyroscope version of dead reckoning. Unfortunately, the package he found is in python. Not in R.

  - Madgwick Orientation Filter: https://ahrs.readthedocs.io/en/latest/filters/madgwick.html

  - However! I found an R version: https://www.rdocumentation.org/packages/RAHRS/versions/1.0.2/topics/MadgwickAHRS

    - CB: “That’s cheating”

    - However, it has basically no documentation and looks kind of dodgy. Going to try it quickly and see what it gives me (but using the information from the better documented document to adjust the variables, etc.)

- Why didn’t gunner just use the gyroscope then???

  - Me: “If the gyroscope is better, why did the ecology papers use the tilt-adjusted accel method in the first place?”

  - CB: “Gyroscopes drift and are more complicated to integrate. When it does work, or when you need it --- like in our case --- it’s more robust… but it is more complicated.”

- SO. It turns out the package I wanted to use has been removed from CRAN because its so broken.  https://cran.r-project.org/web/packages/RAHRS/index.html Therefore I would be forced to revert to using the python version…

  - Before I do this, I’m going to quickly read the other papers though…

    - Reading them confirmed what CB already knew… the animals where there were grazing periods weren’t collared around the neck…

INSERT MY SUMMARY HERE

- Also went back to the beginning of my scripts and ensured that all cleaning and organising was done before they were split into days to make the transfer into python more seamless without data duplication.

#### Moving to Python to use the Madgwick method to replace the tilt-adjusting Gundog.Compass (still use Tracks) for dead reckoning re-orientation

- Divide the gyroscope by 2^15 and then multiply by either [250 or 500] * pi / 180

- After all that, I got:

### 25/05

- Holy heck we’re on a countdown now. UUUGGGHHH. Got to figure this out. I feel slightly numb to the stress of all this because no matter what I try, it just gets worse. Nevertheless, I feel that I must be on the right path now and am at the very least, moving forwards.

- Maybe the axes are wrong? Maybe I have to change which one is the x, y, and z because our device was oriented differently?

  - The Magwick equation assumes NED but I am not in NED. I am in acc = "NWU" and mag = “NED”. (as determined previously by guess and check)

  - Therefore, I need to rotate the axes:

    - X_NED = X_NWU
Y_NED = -Y_NWU
Z_NED = -Z_NWU

- Wait wait. What am I doing. Stop. What is the problem, what do I have, and what am I doing?

  - The problem is that I have an animal that changes orientation during 2 forms of locomotion.

  - Couldn’t I just split the compass section in half?

### 26/05

- Today CB is coming in and will be able to help with the quaternions. He says that the quaternions in the impala data might actually be right. Sucks that I COMPLETELY removed them. Going back to the previous step to reinclude.  I reincluded them and they were trash lmao. I am clearly scaling something completely wrong.

- Until we have run a test in the field, there is no way of knowing whether the tracks are correct or not. All we have is estimate on estimate with rotating collars. We need to do a test in the field. They are soldering a board now, then I will be able to test it in a few days.

- ChrisC said that I should make a speed distribution of the DR track that we calculated so far and see whether we are getting reasonable speeds. If they are reasonable, I am going to call it.

  - Okay, went to make my old method again and it broke completely. Lmao? Back to the beginning to rescale the accels and mags and stuff.

  - I am going back to the very beginning and saving each change as a new column and then sending it to ChrisB to figure out while I’m away.

- God this makes me miserable. I will be so grateful when he finds the solution.

- I need to move on to making the behavioural model predictions for now.

### 27/05

- Today I am working on reverting everything to get us back to where we were before. Upsetting but necessary. Breaking every step into its own function so I can rapidly experiment. I should have done it like this from the beginning on just the minimum section of data. Silly that I didn’t.

- I went and found the data from when everything was working to see what the scales were. Confirmed that I previously used 8192 as the accelerometer division factor.

- Went back to the original data and the original code and got it working. Now I need to increment the old code over to the new code until it works again.

  - Holy heck. So there was something wrong with my scaling and my smoothing logic… Now I get these results… Which essentially brings us back to where I was before. Okay cool. So now I need to correct it forwards from here with these correctly scaled/smoothed values.

- Note that I tried to use the quaternion components onboard but found that they aren’t normalised, which means you can’t calculate the w component (which we don’t have)… so that means we cannnn’t use the on board calculated ones???? Have messaged ChrisB to ask.

  - He responded with a chunk of math… that gave me the following… Which isn’t right but also isn’t completely wrong! Yay!

### 28/05

- Assuming that Birdy will figure out the quaternions while I’m away, my task for now is to get the behavioural model working.

- I got started on generating the features for the impala deployment data collar 8 – intending this to be my example collar for the conference.

- Leaving the quaternions for now and just focusing on what I got with Gundog.Compass. If the speed estimates are reasonable, I’m moving onto behaviours and then finalising for the presentation.

  - The speeds from the moving sections are:

- Min.  1st Qu.   Median     Mean  3rd Qu.     Max.

- 0.00009  0.03132  0.07815  0.18160  0.18677 17.06187

  - This looks reasonable… lets do the next day’s worth of data to see what we get.

https://www.linkedin.com/feed/update/urn:li:groupPost:12555223-7464907944896372736/?utm_source=share&utm_medium=member_desktop&rcm=ACoAAD5PrlYBAzjQErdcyNCGxIzkzzEWB0YxR2M

- Below are the steps I need to take to integrate into the dead reckoning

☐Determine the optimal feature set

☐Generate optimal features across all of the unlabelled data

☐Predict behaviours onto all of the unlabelled data

☐That will take ages, so in the meantime create a simple threshold of head up vs down

☐Separate DR algorithms for when the head is up or down

- Tasks:

  - Convert all the accels to g force and mag units

  - Standardise orientation so all devices roughly the same (make it the same as the top image)

  - Join the cropped 4g accels with the axivity uncropped data to complete the peaks

  - Smooth and filter the data

  - Re-extract all of the training data from the newly standardised data

  - Proceed from there lmao.

## What I learnt and Workflow for next time

- Clean data

- All in the same scale

- Remove misreads and noise