# Aligning Accelerometer with Video Instructions

These are the instructions I wrote for my undergraduate research assistant working on cat and impala data for some of my papers. Path and file names are specific to the project we were working on however the general flow will work on any project.

Contents

Rough Alignment	1

Annotating the Accelerometer Clips	2

Setting Up the Custom Interface	2

Align Video and Accelerometer	3

Annotate Behaviours at Multiple Levels	3

Save and Sanity Check	3

In order to create precise and accurate training data, we require the video of the animal to be synchronised with the accelerometer data from the same animal. The easiest way to do this is to make sure the accelerometer and the camera are set to the same clock before we release the animal. While this seems simple in theory, for some reason, it nearly never actually happens, lmao. There are several ways the two clocks can become desynchronised, e.g.:

- We forget to synchronise them in the first place. (We’re busy and tired… it happens)

- Every device is in a different time zone (i.e., the display time may look the same, but the encoded time is different  this can be a major sneaky problem down the line!).

- The devices start off synchronised but drift due to technological imperfections…

Long story short, it is frequently the case that our videos don’t match with the accelerometers. Therefore, before we can label the accelerometer trace with the behaviours from the video, we must match them up.

If your data is already synchronised, skip ahead to the annotation steps Annotating the Accelerometer Clips. Otherwise, align first in R (because R is faster and easier to play with than matlab, imo).

## Rough Alignment

Information you need:

- The accelerometer, with known times and time zone (e.g., UTC time)

- The videos, with known times and R encoded time zone (e.g., local time, e.g., UTC + 2, South Africa --- SAST, Africa/Johannesburg)

- Conversion between the accelerometer and camera time / time zones (e.g., camera time – 2 = accelerometer time (now both are in UTC))

  - NOTE: It may be the case that every camera has a different setting. E.g., It took us a few hours to realise that Jaz’s camera had a hardcoded daylight savings setting even when daylight savings aren’t appropriate… :0 yikes

- Use the R code Scripts/RoughAlignment/AccelDelayFinder.R

- Extract the times that the video probably happened based on the conversations (if you needed to convert  ps. All our projects will need conversion from local to UTC)

- Plot the probable segment using the shiny app and open the video in another tab

- Play around with the slide bar to adjust when the video starts and ends

- Can you get an alignment with the video? Does it look reasonable? When it looks good, hit save. This will save the exact segment of the accelerometer displayed in the plot, to be used in the next annotation phase :)

## Annotating the Accelerometer Clips

### Setting Up the Custom Interface

- Open the matlab interface (named: Sync_station_vs.m in the matlab app using Mathworks account)

- Open video using the button and file explorer

- All videos will be located within the Project_Name/Raw_Data folder

- Find the animal name (e.g., Clio)

- Find the correct video name (e.g., MAH02857)

- Open

- Open accelerometer data using the button and file explorer

- Accelerometer data will be in the same folder as the video

- Make sure that the accelerometer trace you are opening has the same name as the video (e.g., video MAH02857 will need MAH02857_clipped_accel.csv — these are the matched observations you created earlier)

- Open

- Wait for everything to load

- Depending on the size of the video and file, there might be a little minute for everything to buffer. Just hold tight

- Load in the behaviours

- There should be a Behavioural_labels.csv file of some description in the Project_Name/Raw_Data folder. Click each of the “Load Behaviours” buttons and load in the associated list.

- Ecological behaviours are high-level behaviours / broad classifications

- Biomechanical behaviours are the fine-scale classifications

- NOTE: Ignore the central option (functional behaviours)  these do encode, but don’t display on the plot so can be a bit tricky.

### Align Video and Accelerometer

- Align the video and the accelerometer

- Try to find a time when a very large event happens in the video (e.g., I tap or clap on the device, or a sleeping animal stands up suddenly). This is the event you will now try to find in the accelerometer

- Look in the accelerometer trace display and try to find where you think that event might be. Use the zoom button to get a better look.

  - Click zoom. Left click at the very beginning of the trace once. Left click a short distance into the trace. The display will update to show you only between those two clicks.

- Play back and forth through the video across the defining event and move the accelerometer play line around until it matches the activity in the video. Move this vertical red line using the ‘delay’ button

  - Click ‘Set Delay’. Click on the accelerometer display where you want the line to be.

### Annotate Behaviours at Multiple Levels

- Decide on an annotation

- Observe what is happening in the video. Pause when a behaviour occurs.

- From the menu list, select the best description.

- Type the number into the box and click ‘tag behaviour’.

- Left click on the furthest left of that behaviour

- Left click on the furthest right of that behaviour

- Repeat for each of the behaviours

- Inch forwards through the video, annotating each second

- It is important for my analysis that every second of the video be annotated in sequence with no gaps. When an event occurs that is not a behaviour in the list, you have 2 options.

  - Tag as ‘transitional’ or ‘non-behaviour’

  - Edit the behaviour csv file outside of matlab to add a new behavioural option and then reload the file inside the matlab gui

## Save and Sanity Check

When you’re happy with the annotations, save the file. This will go as “_tagged” to the same file as the clipped accelerometer came from. Check regularly in R or Excel that these sections look reasonable. For reasonability and sanity checking, see next doc!